TOSHIBA PC-Card Controller

TOPIC97

Register Specification

Draft

Rev. 01 1998-02-03

Technical data contained herein are proprietary information of 
Toshiba Corporation which shall be treated confidentially, 
and shall not be furnished to third parties or made public 
without prior written permission by Toshiba Corporation.

There Technical data are subject to export control law of 
Japan/COCOM regulations, and diversion contrary thereto is

prohibited.

TOSHIBA CORPORATION

TOSHIBA CORPORATION

No.

Contents

11	PC-Card IF Configuration Register	4

11.1	Vendor ID Register	4

11.2	Device ID Register	4

11.3	Command Register	4

11.4	Status Register	5

11.5	Revision ID Register	6

11.6	Class Code Register	6

11.7	Header Type Register	6

11.8	Base Address Register	6

11.9	Secondary Status Register	7

11.10	PCI Bus Number Register	8

11.11	Secondary Bus Number Register	8

11.12	Subordinate Bus Number Register	8

11.13	1st Memory window Base Register	8

11.14	1st Memory window Limit Register	8

11.15	2nd Memory window Base Register	9

11.16	2nd Memory window Limit Register	9

11.17	1st I/O window Base(Lower)  Register	9

11.18	1st I/O window Base(Upper) Register	9

11.19	1st I/O window Limit(Lower)  Register	10

11.20	1st I/O window Limit(Upper)  Register	10

11.21	2nd I/O window Base(Lower)  Register	10

11.22	2nd I/O window Base(Upper)  Register	10

11.23	2nd I/O window Limit(Lower) Register	11

11.24	2ndI/O window Limit(Upper) Register	11

11.25	Bridge Control Register	12

11.26	PC Card16 Legacy Mode Base Address Register	14

11.27	ZV Control Register	14

11.28	Slot Control Register	15

11.29	Interrupt Control Register	16

11.30	Card Detect Control Register	17

11.31	CardBus Socket Register Control Register	18

11.32	Wake Up Control Register	20

11.33	Serial Interrupt Control Register	22

11.34	MISC1 Register	22

11.35	MISC2 Register	23

11.36	Power Save Register	24

11.37	Configuration Control Register	25

12	PC-Card16 Register	26

12.1	ID& Revision Register	26

12.2	Interface Status Register	26

12.3	Power Control Register	27

12.4	Interrupt and General Control Register	29

12.5	Card Status Change Register	30

12.6	Card Status Change Interrupt Register	31

12.7	Window Enable Register	32

12.8	I/O Window Control Register	33

12.9	I/O Window 0 Start Low Address Register	34

12.10	I/O Window 0 Start High Address Register	34

12.11	I/O Window 0 End Low Address Register	34

12.12	I/O Window 0 End High Address Register	34

12.13	I/O Window 1 Start Low Address Register	35

12.14	I/O Window 1 Start High Address Register	35

12.15	I/O Window 1 End Low Address Register	35

12.16	I/O Window 1 End High Address Register	35

12.17	System Memory Window 0 Start Low Address Register	36

12.18	System Memory Window 0 Start High Address Register	36

12.19	System Memory Window 0 End Low Address Register	37

12.20	System Memory Window 0 End High Address Register	37

12.21	Memory Window 0 Offset Low Address Register	38

12.22	Memory Window 0 Offset High Address Register	38

12.23	Additional General Control Register	39

12.24	System Memory Window 1 Start Low Address Register	40

12.25	System Memory Window 1 Start High Address Register	40

12.26	System Memory Window 1 End Low Address Register	41

12.27	System Memory Window 1 End High Address Register	41

12.28	Memory Window 1 Offset Low Address Register	42

12.29	Memory Window 1 Offset High Address Register	42

12.30	Global Control Register	43

12.31	System Memory Window 2 Start Low Address Register	44

12.32	System Memory Window 2 Start High Address Register	44

12.33	System Memory Window 2 End Low Address Register	45

12.34	System Memory Window 2 End High Address Register	45

12.35	Memory Window 2 Offset Low Address Register	46

12.36	Memory Window 2 Offset High Address Register	46

12.37	System Memory Window 3 Start Low Address Register	47

12.38	System Memory Window 3 Start High Address Register	47

12.39	System Memory Window 3 End Low Address Register	48

12.40	System Memory Window 3 End High Address Register	48

12.41	Memory Window 3 Offset Low Address Register	49

12.42	Memory Window 3 Offset High Address Register	49

12.43	System Memory Window 4 Start Low Address Register	50

12.44	System Memory Window 4 Start High Address Register	50

12.45	System Memory Window 4 End Low Address Register	51

12.46	System Memory Window 4 End High Address Register	51

12.47	Memory Window 4 Offset Low Address Register	52

12.48	Memory Window 4 Offset High Address Register	52

12.49	PC-Card IF Card Timing Register	53

12.50	Audio/Video Switching Register	54

12.51	PC-Card IF Function Control Register	55

12.52	Toshiba Hidden Register	56

13	Card Bus Register	57

13.1	Socket Event Register	57

13.2	Socket  Mask  Register	58

13.3	Socket Present State Register	59

13.4	Socket Force Register	61

13.5	Power Control Register	63

# PC-Card IF Configuration Register

## Vendor ID Register

| Config | NAME | RW | D16 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- |
| 00 | PPVID | R |  |  | 1179h |

## Device ID Register

| Config | NAME | RW | D31 | D16 | PCLR |
| --- | --- | --- | --- | --- | --- |
| 02 | PPDID | R |  |  | 60Fh |

## Command Register

| Config | NAME | RW | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 04 | PPCMD | RW | - | - | - | - | - | - | MFBE | SRRE | 0000h |
|  |  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |  |
|  |  |  | WCC | PERE | - | MWIE | SCE | BME | MAE | IOAE |  |

D9:		Fast Back-to-Back Enable ( “0” fixed )

This field is fixed “0”. Because a master can not do fast back-to-back transactions to different devices.

D8:		SEER# Enable (“0” fixed )

This field is fixed “0”. The SERR# output buffer is disabled even if the Parity Error happened.

D7:		Wait Cycle Control (“0” fixed )

This field is fixed “0”. Because a address/data stepping is not supported.

D6:		Parity Error Response (“0” fixed )

This field is fixed “0”. The SERR#/PERR# output buffer is disabled even if the Parity Error happened.

D4:		Memory write (“0” fixed )

This field is fixed “0”. Every memory write operation is carried out by the Memory Write Command.

D3:		Special Cycle (“0” fixed )

This field is fixed “0”. Because a response to Special Cycle operations is not supported.

D2:		Bus Master

Enable to act as a master on the PCI Bus and CardBus

1:	Possible to act as a master on the PCI Bus and CardBus.

0:　	Impossible to act as a master on the PCI Bus and CardBus. Responds to the access from PC card for CardBus by master-abort.

D1:		Memory Space

1:	Possible to respond to memory space accesses.

0:	Impossible to respond to memory space accesses.

D0:		I/O Spcae

1:	Possible to respond to I/O space accesses.

0:	Impossible to respond to I/O space accesses.

## Status Register

| Config | NAME | RW | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 06 | PPST | RW | DPER | SSER | RMAB | RTAB | STAB | DVT1 | DVT0 | DPDT | 0480h |
|  |  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |  |
|  |  |  | FBBE | - | - | - | - | - | - | - |  |

D15:		Detected Parity Error

This field is set to “1” if this controller detect a parity error when it acts as a master or a target on the PCI Bus.

D14:		Signaled System Error

This field is set to “1” if this controller signal that it asserts SERR# on the PCI Bus.

D13:		Received Master Abort

This field is set to “1” if this controller terminate its transaction with master-abort when it acts as a master on the PCI Bus.

D12:		Received Target Abort

This field is set to “1” if this controller is terminated its transaction by target-abort when it acts as a master on the PCI Bus.

D11:		Signaled Target Abort

This field is set to “1” if this controller terminates a transaction with target-abort when it acts as a master on the PCI Bus.

D10-D9:	DEVSEL# Timing

0 0:  Fast

0 1:  Medium

1 0:  Slow

1 1:  sub

D8:		Data Parity Detected

This filed is set to “1” by the following conditions.

- This controller asserted PERR# itself or observed PERR# asserted.

- PERR# asserted when this controller acts as a master on the PCI Bus.

・When D6 of Command Register(Config:04h) is set to “1”.

D7:		Fast Back-to-Back Capable ( “1” fixed )

1:　Enable

0:　Disable

D15-D11 and D8 are reset to “0” by writing “1”.

## Revision ID Register

| Config | NAME | RW | D7 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- |
| 08 | PPRID | RW |  |  | 06h |

## Class Code Register

| Config | NAME | RW | D31 | D8 | PCLR |
| --- | --- | --- | --- | --- | --- |
| 09 | PPCC | RW |  |  | 060700h |

## Header Type Register

| Config | NAME | RW | D23 | D16 | PCLR |
| --- | --- | --- | --- | --- | --- |
| 0E | PPHT | R |  |  | 82h |

## Base Address Register

| Config | NAME | RW | D31 |  |  | D16 | D15 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | PPBA | RW | BA31-16 |  |  |  | - |  | - | 0h |

D31-D16:	Base Address

Sets up the base-address for mapping a Register-set of PC-Card16 and CardBus into the system resource. The Register-set of PC-Card16 and CardBus is mapped into 4K Byte memory fields from this base address.

The CardBus Register-set is mapped between 7FFh from 000h of the 4K Byte memory field. The PC-Card16 Register-set is mapped between FFFh from 800h of the 4K Byte memory field. (The PC-Card16 Register-set is mapped at the head in the address which added the offset value in PC-Card16 Register to 800h.)

| Base Address | CardBus Register Set | 000h 7FFh |
| --- | --- | --- |
|  | PC-Card16 Register Set | 800h FFFh |

## Secondary Status Register

| Config | NAME | RW | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | PPCMD | RW | DPER | RSER | RMAB | RTAB | STAB | DVT1 | DVT0 | DPDT | 0480h |
|  |  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |  |
|  |  |  | FBBE | - | - | - | - | - | - | - |  |

D15:		Detected Parity Error

This field is set to “1” if this controller detect a parity error when it acts as a master or a target on the CardBus.

D14:		Received System Error

This field is set to “1” if this controller receive that SERR# is asserted on the CardBus.

D13:		Received Master Abort

This field is set to “1” if this controller terminate its transaction with master-abort when it acts as a master on the CardBus.

D12:		Received Target Abort

This field is set to “1” if this controller is terminated its transaction by target-abort when it acts as a master on the CardBus.

D11:		Signaled Target Abort

This field is set to “1” if this controller terminates a transaction with target-abort when it acts as a master on the CardBus.

D10-D9:	DEVSEL# Timing

0 0:  Fast

0 1:  Medium

1 0:  Slow

1 1:  sub

D8:		Data Parity Detected

This filed is set to “1” by the following conditions.

- This controller asserted PERR# itself or observed PERR# asserted.

- PERR# asserted when this controller acts as a master on the PCI Bus.

・When D0 of Bridge Control Register(Config:3Eh) is set to “1”.

D7:		Fast Back-to-Back Capable ( “1” fixed )

1:　Enable

0:　Disable

D15-D11 and D8 are reset to “0” by writing “1”.

## PCI Bus Number Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | PPPBN | RW | BNO7 | BNO6 | BNO5 | BNO4 | BNO3 | BNO2 | BNO1 | BNO0 | 00h |

D7-D0:	PCI Bus Number

This register holds the number of the PCI Bus.

## Secondary Bus Number Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | PPSCN | RW | CNO7 | CNO6 | CNO5 | CNO4 | CNO3 | CNO2 | CNO1 | CNO0 | 00h |

D7-D0:	CardBus Bus Number

This register holds the number of the CardBus attached to the socket.

## Subordinate Bus Number Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1A | PPSBN | RW | SNO7 | SNO6 | SNO5 | SNO4 | SNO3 | SNO2 | SNO1 | SNO0 | 00h |

D7-D0:	Subordinate CardBus Bus Number

This register holds the number of the CardBus at lowest part of the hierarchy when the CardBus attached hierarchically.

## 1st Memory window Base Register

| Config | NAME | RW | D31 |  |  |  | D12 | D11 |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1C | PP1MB | RW | AD31 |  |  |  | AD12 | - |  |  | 00000h |

D31-D12:	1stMemory Window Base Address

This register holds a base address for forwarding a memory transaction on the PCI Bus to the CardBus.

It defines AD31-12 on the CardBus. The lower 12 bits are all fixed “0”.

The memory space on the CardBus is starting on a 4K boundary.

## 1st Memory window Limit Register

| Config | NAME | RW | D31 |  |  |  | D12 | D11 |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | PP1ML | RW | AD31 |  |  |  | AD12 | - |  |  | 00000h |

D31-D12:	1stMemory Window Limit Address

This register holds a limit address for forwarding a memory transaction on the PCI Bus to the CardBus.

It defines AD31-12 on the CardBus. The lower 12 bits are all fixed “1”, but they are all set to “0” when a read access.

If the AD31-12 of this register are all set to “0”, any memory access is not accepted.

## 2nd Memory window Base Register

| Config | NAME | RW | D31 |  |  |  | D12 | D11 |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | PP2MB | RW | AD31 |  |  |  | AD12 | - |  |  | 00000h |

D31-D12:	2ndMemory Window Base Address

This register holds a base address for forwarding a memory transaction on the PCI Bus to the CardBus.

It defines AD31-12 on the CardBus. The lower 12 bits are all fixed “0”.

The memory space on the CardBus is starting on a 4K boundary.

## 2nd Memory window Limit Register

| Config | NAME | RW | D31 |  |  |  | D12 | D11 |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | PP2ML | RW | AD31 |  |  |  | AD12 | - |  |  | 00000h |

D31-D12:	2ndMemory Window Limit Address

This register holds a limit address for forwarding a memory transaction on the PCI Bus to the CardBus.

It defines AD31-12 on the CardBus. The lower 12 bits are all fixed “1”, but they are all set to “0” when a read access.

If the AD31-12 of this register are all set to “0”, any memory access is not accepted.

## 1st I/O window Base(Lower)  Register

| Config | NAME | RW | D15 |  |  |  |  | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2C | PP1IBL | RW | AD15 |  |  |  |  | AD2 | - | - | 0000h |

D15-D2:	1stI/O Window Base Address (Lower)

This register holds a lower 16 bits of a base address for forwarding a I/O transaction on the PCI Bus to the CardBus.

It defines AD15-2 on the CardBus. The lower 2 bits of this register are all fixed “0”.

The I/O space on the CardBus is starting on a 4K boundary.

## 1st I/O window Base(Upper) Register

| Config | NAME | RW | D15 |  |  |  |  |  |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2E | PP1IBU | R | AD31 |  |  |  |  |  |  | AD16 | 0000h |

D15-D0:	1stI/O Window Base Address (Upper) ( “0000h” fixed )

This register holds a upper 16 bits of a base address for forwarding a I/O transaction on the PCI Bus to the CardBus.

This register are fixed “0000h”.

It defines AD31-16 on the CardBus.

## 1st I/O window Limit(Lower)  Register

| Config | NAME | RW | D15 |  |  |  |  | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | PP1ILL | RW | AD15 |  |  |  |  | AD2 | - | - | 0000h |

D15-D2:	1stI/O Window Limit Address (Lower)

This register holds a lower 16 bits of a limit address for forwarding a I/O transaction on the PCI Bus to the CardBus.

It defines AD15-2 on the CardBus. The lower 2 bits of this register are all fixed “1”.

If the AD15-2 of this register are all set to “0”, any I/O access is not accepted.

## 1st I/O window Limit(Upper)  Register

| Config | NAME | RW | D15 |  |  |  |  |  |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | PP1ILU | R | AD31 |  |  |  |  |  |  | AD16 | 0000h |

D15-D0:	1stI/O Window Limit Address (Upper) ( “0000h” fixed )

This register holds a upper 16 bits of a limit address for forwarding a I/O transaction on the PCI Bus to the CardBus.

This register are fixed “0000h”.

It defines AD31-16 on the CardBus.

## 2nd I/O window Base(Lower)  Register

| Config | NAME | RW | D15 |  |  |  |  | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 | PP2IBLL | RW | AD15 |  |  |  |  | AD2 | - | - | 0000h |

D15-D2:	2ndI/O Window Base Address (Lower)

This register holds a lower 16 bits of a base address for forwarding a I/O transaction on the PCI Bus to the CardBus.

It defines AD15-2 on the CardBus. The lower 2 bits of this register are all fixed “0”.

The I/O space on the CardBus is starting on a 4K boundary.

## 2nd I/O window Base(Upper)  Register

| Config | NAME | RW | D15 |  |  |  |  |  |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | PP2IBU | R | AD31 |  |  |  |  |  |  | AD16 | 0000h |

D15-D0:	2ndI/O Window Base Address (Upper) ( “0000h” fixed )

This register holds a upper 16 bits of a base address for forwarding a I/O transaction on the PCI Bus to the CardBus.

This register are fixed “0000h”.

It defines AD31-16 on the CardBus.

## 2nd I/O window Limit(Lower) Register

| Config | NAME | RW | D15 |  |  |  |  | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38 | PP2ILL | RW | AD15 |  |  |  |  | AD2 | - | - | 0000h |

D15-D2:	2ndI/O Window Limit Address (Lower)

This register holds a lower 16 bits of a limit address for forwarding a I/O transaction on the PCI Bus to the CardBus.

It defines AD15-2 on the CardBus. The lower 2 bits of this register are all fixed “1”.

If the AD15-2 of this register are all set to “0”, any I/O access is not accepted.

## 2ndI/O window Limit(Upper) Register

| Config | NAME | RW | D15 |  |  |  |  |  |  | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3A | PP2ILU | R | AD31 |  |  |  |  |  |  | AD16 | 0000h |

D15-D0:	2ndI/O Window Limit Address (Upper) ( “0000h” fixed )

This register holds a upper 16 bits of a limit address for forwarding a I/O transaction on the PCI Bus to the CardBus.

This register are fixed “0000h”.

It defines AD31-16 on the CardBus.

## Bridge Control Register

| Config | NAME | RW | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3E | PPBCN | RW | - | - | - | - | - | WBEN | M1PE | M0PE | 0300h |
|  |  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |  |
|  |  |  | INTE | CBRS | MSAM | - | VGAE | ISAE | SERR | PERE |  |

D10:		Write Buffer Enable

1:  Enable

Write buffers are enabled

0:  Disable

D9:		Memory1 Pre-fetch Enable

1:  Enable

Pre-fetch operation is carried out when a memory read transaction to the 2ndMemory Window space.

0:  Disable

D8:		Memory0 Pre-fetch Enable

1:  Enable

Pre-fetch operation is carried out when a memory read transaction to the 1stMemory Window space.

0:  Disable

D7:		IRQ-INT Enable

1:  Enable

The interrupt signal from Card (BSYAZ/BSYBZ) is assigned to one of the serial interrupts signals (IRQ3-15) in case of PC-Card16/CardBus.

The assignment is determined according to D3-0 of Interrupt and General Control Register(Offset:03h).

0:  Disable

The interrupt signal from Card (BSYAZ/BSYBZ) is assigned to INTAZ/INTB in case of PC-Card16/CardBus.

The assignment is determined according to D5-4 of Interrupt Control Register(Offset:A1h).

|  | PC-Card16 IF |  | PC-Card16/CardBus IF |  |
| --- | --- | --- | --- | --- |
|  | PC-Card16 | CardBus | PC-Card16 | CardBus |
| Card Interrupt | IRQx | - | @2 IRQx / INTx |  |
| Status change Interrupt | @1 IRQx / INTx | - | INTx | INTx |

@1: Switched by IREQ/INT Select bit of PC-Card16 Socket Control Register.

@2: Switched by IREQ-INT Enable bit of Bridge Control Register.

D6:		CardBus Reset Enable

1:  Enable

CRST# is enable to assert on the CardBus.

0:  Disable

D5:		Master Abort mode

This field control to signal a target abort during a master abort asserted.

1:  Possible to signal a target abort.

0:  Impossible to signal a target abort.

D3:		VGA Enable

1:  Enable

Possible to forward a transaction of VGA on the PCI Bus to the CardBus  through the I/O space (3B0h-3BBh, 3C0h-3DFh) and the memory space (A0000h-BFFFFh). But a transaction to the PCI Bus from the CardBus is inhibited by a master abort.

Configuration Register (in which I/O space and memory space and ISA enable are set up) is ineffective at this condition.

0:  Disable

D2:		ISA Enable

1:  Enable

In the access to the CardBus from the PCI Bus, the transaction is forwarded to the lower 256 bytes  of I/O space of the 1 K-byte unit defined by the I/O window BaseXX register - the I/O window LimitXX register of a Configuration.

In the access to the PCI Bus from the CardBus, the transaction is forwarded to the upper 768 bytes  of I/O space of the 1 K-byte unit defined by the I/O window BaseXX register - the I/O window LimitXX register of a Configuration.

0:  Disable

The transaction is forwarded to the I/O space defined by the I/O window BaseXX Register – the I/O window LimitXX Register.

D1:		SERR# Status Enable

1:  Enable

SEER# signaled on the CardBus is related to D14 of Status Register(Config:06h), only when the D8 of Command Register(Config:04h) is set to “1”.

0:  Disable

D0:		Parity Error Response Enable

1:  Enable

Possible to signal and detect an address/data parity error signaled on the CardBus.

0:  Disable

## PC Card16 Legacy Mode Base Address Register

| Config | NAME | RW | D31 |  |  | D16 | D15 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44 | PPLMB | RW | - |  |  |  | LMB15-1 |  | - | 01h |

D15-D1:	PC-Card16 Legacy Mode Base Address

This register holds a base address with Legacy Mode when Card Detect Control Register(Config:A3h) is set to “0”.

Is supports a Legacy Mode operation with non-zero address.

D0:		Unused (“1” fixed)

## ZV Control Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9C | PPZVC | RW | - | - | - | - | - | - | - | ZVCN | 00h |

D0:		ZV Port Control Enable

1:  Enable

AUDIO/VIDEO ZV port function is enabled.

0:  Disable

Possible to read a status of ZV port when D1 of MISC2 Register(Config:AEh) is set to “1”. This bit is set to “1” if ZV port is used.

It changes when D1 or D0 of Audio/Video switching Register(Offset:3Ch) is changed, since they are related to each other.

## Slot Control Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A0 | PPSLT | RW | ESLT | EREG | EHDN | IDWP | PAD1 | PAD0 | OFS1 | OFS0 | 00h |

D7:		SLOTON

1:  Enable

CardBus slot function is enabled.

0:　Disable

D6:		 SLOTEN

1:  Enable

The access to the register-set of PC-Card16 and CardBus is enabled.

0:　Disable

D5:		ID_LOCK

1:　Disable

The write access to Toshiba Hidden Register(Offset:3Fh) is enabled.

0:  Enable

D4:		ID_WP

1:  Enable

The write access to ID & Revision Register(Offset:00h) is enabled.

0:　Disable

D3-D2:	Slot Port Select

0 0:  03E0h/03E1h

0 1:  03E2h/03E3h

1 0:  03E4h/03E5h

1 1:  03E6h/03E7h

D1-D0:	Offset Select

0 0:  00h-3Fh

0 1:  40h-7Fh

1 0:  80h-BFh

1 1:  C0h-FFh

## Interrupt Control Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | PPIRC | RW | - | - | IPA1 | IPA0 | - | STIQ | IRPL | IRLN | 00h |

D5-D4:	Interrupt Pin Assign

These fields assign an interrupt to an external output signal(INTAZ/INTBZ).

0 0:  Disable

0 1:  INTAZ

1 0:  INTBZ

1 1:  -

D2:		STSIRQNP

This field set up an output mode of a status change interrupt.

1:  Pulse mode

0:　Trough mode

D1:		IRQNP

This field set up an output mode of an IRQ interrupt.

1:  Pulse mode

0:　Trough mode

D0:		IRQ/INT Select

1: INTA Selected

The status change interrupt(BVDAZ/BVDBZ) from Card is assigned to INTAZ/INTBZ in case of PC-Card16.

The assignment is determined according to D5-4 of Interrupt Control Register(Config:A1h).

0:  IRQ Selected

The status change interrupt(BVDAZ/BVDBZ) from Card is assigned to INTAZ/INTBZ in case of PC-Card16.

The assignment is determined according to D3-0 of Interrupt & General Control Register(Offset:03h).

## Card Detect Control Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3 | PPCDT | RW | PCMD | - | - | - | - | VSS1 | VSS0 | DMSL | 00h |

D7:		PC-Card16/CardBus mode Select

1:  PC-Card16/CradBus mode

0:  PC-Card16 mode

D2-D1:	Voltage Sense Pin Enable

The external output signal VS2AZ/VS2BZ and VS1AZ/VS1BZ are enabled when a Card is detected by software( D0 of this register is set to “1”.)

0 0:  VS2AZ/VS2BZ=”L”,VS1AZ/VS1BZ=”L”

0 1:  VS2AZ/VS2BZ=”L”,VS1AZ/VS1BZ=”Z”

1 0:  VS2AZ/VS2BZ=”Z”,VS1AZ/VS1BZ=”L”

1 1:  VS2AZ/VS2BZ=”Z”,VS1AZ/VS1BZ=”Z”

D0:		Detect Method Select

1:  A Card is detected by software

0:  A Card is detected by Hardware

## CardBus Socket Register Control Register

| Config | NAME | RW | D31 | D30 |  | D29 |  | D28 |  | D27 |  | D26 | D25 | D24 |  | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A4 | PPSRC | RW | RRE | CRRE |  | CKRN |  | CTST |  | - |  | - | BSPO | BSSO |  | 00h |
|  |  |  | D23 |  |  |  |  |  | D16 | D15 D11 |  |  | D10 |  | D8 | PCLR |
|  |  |  | - |  |  |  |  |  |  | CDD4-0 |  |  | - |  |  | 0000h |
|  |  |  | D7 |  |  |  |  |  |  |  | D3 | D2 | D1 | D0 |  | PCLR |
|  |  |  | - |  |  |  |  |  |  |  |  | DRI | CAOF | CAIE |  | 00h |

D31:		Resum Reset Enable

1:  Enable

The CardBus Register-set are reset when resumed.

0:  Disable

The CardBus Register-set are kept when resumed.

D30:		CardBus Card Removal Reset Enable

1:  Enable

The CardBus Register-set are reset when CardBus is removed.

0:  Disable

D29:		CardBus CLOCKRUN Enable

1:  Enable

The CLOCKRUN function of CardBus is enabled.

It has priority over D7 of Power ControlレRegister (Offset:10h).

0:  Disable

D28:		H256PCTE

1:  Test mode

The 256PCI counter is used as 16PCI counter.

0:  Normal mode

D27:		IOPLUP

1:  The IO cell in the CardBus side are pulled up.

0:  Normal mode.(IO cell are not pulled up)

D25:		BUFOFF Socket Power Off

1:  Power to the socket is turned off in case of Buffer off.

0:  Power to the socket is supplied in case of Buffer off.

D24:		BUFOFF Socket Signal Off

1:  Signal to the socket is set to “HZ” in case of Buffer off.

0:  Signal to the socket is output in case of Buffer off.

D15-D11:	CardBus Device Number

These fields hold the CardBus Device Number.

D2:		RI Disable

1:  Disable

PC Card Ring is disabled.

0:  Enable

D1:		CAUDIO Signal OFF Enable

1:  Enable

External input BVD2A/BVD2B(CardBus: CAUDIO) is invalid.

0:  Disable

D0:		CAUDIO Invert Enable

1:  Enable

External input signal BVD2A/BVD2B (CardBus: CAUDIO) is inverted.

0:  Disable

External output signal AUDIO is controlled with CAOF and CAIE.

| PC Card Type | CAOF CAIE | AUDIO |
| --- | --- | --- |
| CardBus | 0 0 0 1 | CAUDIO ~CAUDIO |
| PC-Card16 | 0 0 0 1 | 0 1 |
| X | 1 0 1 1 | 0 1 |

## Wake Up Control Register

| Config | NAME | RW | D31 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A8 | PPWUC | RW | - |  | EC2W | EC1W | EB3W | EB2W | EB1W | EB0W | 00h |
|  |  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
|  |  |  | ESCW | EIRW | EITW | ECSW | ESPS | ECLK | ECHG | SCHG | 00h |

D31-D14:	Unused (R/W)

D13:		CD2AZ/CD2BZ Pin Wake-Up Enable

1:  Enable

Wake-Up by external input signal CD2AZ/CD2BZ is enabled.

0:  Disable

D12: 		CD1AZ/CD1BZ Pin Wake-Up Enable

1:  Enable

Wake-Up by external input signal CD1AZ/CD1BZ is enabled.

0:  Disable

D11: 		Socket Event Register D3 Wake-Up Enable

1:  Enable

Wake-Up by D3(Power Cycle) of Socket Event Register(Offset:000h) is enabled.

0:  Disable

D10: 		Socket Event Register D2 Wake-Up Enable

1:  Enable

Wake-Up by D2(CCD2Z) of Socket Event Register is enabled.

0:  Disable

D9: 		Socket Event Register D1 Wake-Up Enable

1:  Enable

Wake-Up by D1(CCD1Z) of Socket Event Register is enabled.

0:  Disable

D8: 		Socket Event Register D0 Wake-Up Enable

1:  Enable

Wake-Up by D0(CSTSCHG/WAKEUP) of Socket Event Register is enabled.

0:  Disable

D7: 		R2 BVD1AZ/BVD1BZ Enable

1:  Enable

Wake-Up by external input signal BVD1AZ/BVD1BZ(STCHGZ) is enabled.

0:  Disable

D6: 		R2 BSYAZ/BSYBZ Enable

1:  Enable

Wake-Up by external input signal BSYAZ/BSYBZ(IREQAZ/IREQBZ) is enabled.

0:  Disable

D5: 		CardBus BSYAZ/BSYBZ Enable

1:  Enable

Wake-Up by external input signal BSYAZ/BSYBZ(CINTZ) is enabled.

0:  Disable

D4: 		CardBus BVD1AZ/BVD1BZ Enable

1:  Enable

Wake-Up by external input signal BVD1AZ/BVD1BZ(CSTSCHG) is enabled.

0:  Disable

D3:		Socket Present Status Register Clear Enable

1:  Enable

Socket Present Status Register is cleared by Card removal in case of Suspend mode.

0:  Disable

D2:		CLK32 Socket Event Register Enable

1:  Enable

Socket Event Register with CLK32 is used.

D7-D4 is enabled.

0:  Disable

D1:		STSCHG Pin Output Enable

1:  Enable

Internal output signal PCSTCG is enabled.

External output signal STSCHGZ is enabled (set to “L”) when D4 of Enable Register(I/O:00EFh、index:6Ch) is “1”.

0:  Disable

D0:		STSCHG Pin Status

This field indicates condition of external output signal STSCHGZ.

It is reset to “0” when written “1”.

## Serial Interrupt Control Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AC | PPSIC | RW | - | - | - | - | - | ISAM | ISSM | SICM | 00h |

D7:		Serial Interrupt SMI Mapping

1:  Interrupt signal is output from Serial interrupt signal.

0:  Direct output mode.

D7:		Serial Interrupt INT Mapping

1:  Interrupt signal is output from Serial interrupt signal.

0:  Direct output mode.

D7:		Interrupt Control Mode Select

1:  Standard Method

0:  Toshiba Method

## MISC1 Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AD | PPM1S | RW | CLKP | CLKM | - | DEMD | - | CSNC | - | RDLE | 00h |

D7:		CLOCKRUN Control Enable

1:  Enable

CLOCKRUN Protocol in PCI side is enabled.

0:  Disable

D6:		CLOCKRUN mode

1:  Clock input is always requested when CardBus Card is inserted.

0:  Clock input is requested only when a transaction is not finished.

D4:		Detect REQ mode Enable

1:  Enable

CLOCKRUN is requested by an interruption while detecting a Card.

0:  Disable

D2:		Socket Present Register clear Disable

1:  Disable

0:  Enable

Socket Present Register(Config:008h) is initialized.

D0:		R2 Data Bus Low Enable

1:  Data Bus is fixed to “00h”.

0:  Data of system is output directly.

## MISC2 Register

| Config | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AE | PPM2S | RW | - | SPC2 | SPC1 | SPC0 | SPCM | SPE | ZVMD | ZVEN | 00h |

D6-D4:	SPWRCLK

000 : 1 * PCICLK

001 : 2 * PCICLK

010 : 4 * PCICLK

011 : 8 * PCICLK

100 : 16* PCICLK

101 : 32* PCICLK

11x : Reserved

D3:		SPWRMOD

1:  Standard method

0:  Toshiba’s method

D2:		SPWR Enable

1:  Enable

0:  Disable

D1:		ZV mode Select

1:  This field enable for indicating the status of ZV Port.

D0 of ZV Control Register(Config:9Ch) and D1-D0 of Audio/Video Switching Register(Offset:3Ch) indicate the status of ZV Port when read.

0:  This field disable for indicating the status of ZV Port.

D0:		ZV Port Enable

1:  Enable

ZV Port is enabled.

0:  Disable

## Power Save Register

| Config | NAME | RW | D31 |  |  |  |  | D10 | D9 | D8 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BC | PPPWS | RW | - |  |  |  |  |  | CCC1 | CCC0 | 00h |
|  |  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
|  |  |  | CKEN | - | DSIR | DIRQ | - | - | - | DVBF | 00h |

D9-D7,D5-D4,D0:	Power Save Register

All 1:  Power Save mode.

All 0:  Normal mode.

## Configuration Control Register

| Config | NAME | RW | D31 |  |  |  |  |  |  | D16 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FC | PPCNF | RW | - |  |  |  |  |  |  |  | 0000h |
|  |  |  | D15 |  |  |  |  |  |  | D8 | PCLR |
|  |  |  | - |  |  |  |  |  |  | - | 00h |
|  |  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
|  |  |  | - | - | - | - | - | - | - | IDWE | 00h |

D0:		ID Register Write Protect Disable

1:  Disable

Write access to the following PC-Card IF Configuration Registers is allowed.

Vendor ID Register

Device ID Register

Revision ID Register

Class Code Register

Subsystem Vendor ID Register

Subsystem Device ID Register

(Config:00h)

(Config:02h)

(Config:06h)

(Config:09h)

(Config:40h)

(Config:42h)

0:  Enable

Only read access is allowed.

# PC-Card16 Register

## ID& Revision Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | 800 | PPRVN | RW | ID1 | ID0 | - | - | REV3 | REV2 | REV1 | REV0 | 00h |

D7-D6:	PCMCIA Card Type ID

D3-D0:	 Revision

This register doesn’t affect PC Card.

This register can write when D4 (Slot Control Register (Config: A0h)) is set to 0.

## Interface Status Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 801 | PPIFS | R | - | CPA | BSYZ | WP | CD2Z | CD1Z | BVD2 | BVD1 | - |

D7:		Unused (1 fixed)

D6:		PC Card Power Active

1:   Card Power Supply  (5V) provided

0:   Card Power Supply  (5V) not provided

D5:		BSYZ

Indicates the status of an external input signal BSYAZ/BSYBZ

D4:		WP

Indicates the status of an external input signal WPA/WPB.

D3:		CD2Z

Indicates the status of an external input signal CD2AZ/CD2BZ.

D2:		CD1Z

Indicates the status of an external input signal CD1AZ/CD1BZ.

D1:		BVD2

Indicates the status of an external input signal BVD2A/BVD2B.

D0:		BVD1

Indicates the status of an external input signal BVD1A/BVD1B.

Indicates 1 when BSYZ, WP and BVD2 Card weren’t inserted.

## Power Control Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | 802 | PPPCN | RW | COE | DRCL | EAPS | CP5E | CP3E | - | VPP1 | VPP0 | 00h |

D7:		 Card Output Enable

1:  Enable

The external output signals (including the signal for both directions) of Slot Interface are made HZ status excepting Card Control Signal(VS2AZ/VS2BZ, VS1AZ/VS1BZ, WPA/WPB, WAITAZ/WAITBZ, RSTA/RATB).

0:  Disable

D6:		Resume Reset Disable

1:  Disable

0:  Enable

PC-Card16 Register is initiated by Resume Reset (an external input signal RCLR =“H”).

D5:		Auto Power Switch Enable

1:  Enable

Card Control Signal enabled when the card is put in.

0:  Disable

D4:		 Card Power (5V) Enable

Card Power Supply  (VCC) enabled.

1:  5V

0:  0V

D3:		 Card Power (3.3V) Enable

Card Power Supply (3.3V) enabled.

“CP5E=1” is extended, and 3.3V enabled when bit0 is set to 1 in PC-Card IF Function Control Register (Offset: 3Eh).

1:  3.3V

0:  5V

| Card Power Enable | Power Control Register |  |  |
| --- | --- | --- | --- |
| 3Eh D0 | D4 | D3 | Power Supply Mode |
| 0 | 0 | － | 0 v |
| 0 | 1 | － | 5 v |
| 1 | 0 | － | 0 v |
| 1 | 1 | 0 | 5 v |
| 1 | 1 | 1 | 3 v |

*D4,3 compatible with 82092AA.

D1-D0:	VPP Control

Slot Power Supply  (VPP) controlled.

| D1 | D0 | Vpp Power Supply Mode |
| --- | --- | --- |
| 0 | 0 | 0.0 v |
| 0 | 1 | 5.0 v／3.3 v |
| 1 | 0 | 12 v |
| 1 | 1 | Reserved |

PC-Card IF Function Control Register (Offset: 3Eh) D0=0

| CARD ON* | Power Control Register(Offset: 02h) |  |  |  |  | Power Supply Provided |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 4 | 3 | 1 | 0 | Vcc | Vpp |
| x | x | 0 | x | x | x | 0 | 0 |
| x | 0 | 1 | x | 0 | 0 | 5 | 0 |
|  |  |  |  | 0 | 1 |  | 5 |
|  |  |  |  | 1 | 0 |  | 12 |
|  |  |  |  | 1 | 1 |  | 0 |
| 1 | 1 | 1 | x | 0 | 0 | 5 | 0 |
|  |  |  |  | 0 | 1 |  | 5 |
|  |  |  |  | 1 | 0 |  | 12 |
|  |  |  |  | 1 | 1 |  | 0 |
| 0 | 1 | 1 | x | x | x | 0 | 0 |

PC-Card IF Function Control Register (Offset: 3Eh) D0=1

| CARD ON* | Power Control Register(Offset: 02h) |  |  |  |  | Power Supply Provided |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 4 | 3 | 1 | 0 | Vcc | Vpp |
| x | x | 0 | x | x | x | 0 | 0 |
| x | 0 | 1 | 0 | 0 | 0 | 5 | 0 |
|  |  |  |  | 0 | 1 |  | 5 |
|  |  |  |  | 1 | 0 |  | 12 |
|  |  |  |  | 1 | 1 |  | 0 |
| x | 0 | 1 | 1 | 0 | 0 | 3.3 | 0 |
|  |  |  |  | 0 | 1 |  | 3.3 |
|  |  |  |  | 1 | 0 |  | 12 |
|  |  |  |  | 1 | 1 |  | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 5 | 0 |
|  |  |  |  | 0 | 1 |  | 5 |
|  |  |  |  | 1 | 0 |  | 12 |
|  |  |  |  | 1 | 1 |  | 0 |
|  | 1 | 1 | 1 | 0 | 0 | 3.3 | 0 |
|  |  |  |  | 0 | 1 |  | 3.3 |
|  |  |  |  | 1 | 0 |  | 12 |
|  |  |  |  | 1 | 1 |  | 0 |
| 0 | 1 | 1 | x | x | x | 0 | 0 |

* CARD ON indicates the status, which the Card put in when, set to 1.

## Interrupt and General Control Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 03 | 803 | PPICN | RW | ERI | CRST | CTYP | ESMI | PIR3 | PIR2 | PIR1 | PIR0 | 00h |

D7:		RI Enable

1:  Enable

The status of signal (an external input signal BVD1A/BVD1B) is the source of Wake-up Event when bit (CTYP( I/O Card Mode) is set to 1

0:  Disable

D6:		 Card Reset Disable

1:  Disable

0:  Enable

The signal (an external output signal RSTA/RSTB) (=“H”) enabled.

D5:		 Card Type

1:  I/O Card Mode

0:  Memory Card Mode

D4:		SMI Enable

1:  Enable

Uses the interrupt by Card Status Change (an external input signal BVD1A/BVD1B) as SMI.

0:  Disable

Assigns the interrupt by D7-4 (Card Status Change Interrupt Register (Offset: 05h)), and uses as IRQ.

D3-D0:	I/O Card Interrupt Level

Assigns the interrupt (an external input signal BSYAZ/BSYBZ) in I/O Card Mode to the interrupt input of PIC.

0 0 0 0:  Disable

0 0 0 1:  Disable

0 0 1 0:  Disable

0 0 1 1:  IRQ3

0 1 0 0:  IRQ4

0 1 0 1:  IRQ5

0 1 1 0:  Disable

0 1 1 1:  IRQ7

1 0 0 0:  Disable

1 0 0 1:  IRQ9

1 0 1 0:  IRQ10

1 0 1 1:  IRQ11

1 1 0 0:  IRQ12

1 1 0 1:  Disable

1 1 1 0:  IRQ14

1 1 1 1:  IRQ15

## Card Status Change Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 04 | 804 | PPCSC | RW | - | - | - | - | CDC | RDYC | BATW | BATD | 00h |

D3:		 Card

1:  Changed

Set up when the Card is pulled out (CD1AZ/CA1BZ and CD2AZ/CD2BZ are changed from ”LL” to ”LH” or ”HL”), or is put in (changed from “HH” to ”LL”). But cannot be sensed as to Detect during suspending.

0:  Not changed

D2:		Ready

1:  Changed

Card Status Change will occur due to external input signal BSYAZ/BSYBZ switched to ”H” from “L” in case of Memory Card

0:  Not changed

In either case of Memory Card whose status is not changed or I/O Card.

D1:		Battery Warning

1:  Battery Warning

Set up in case of Memory Card Battery Warning (an external input signal BVD1A/BVD1B and BVD2A/BVD2B are changed from ”HH” to ”HL”).

Set up once, or never be 0 even if returned to Battery Good (“HH”).

0:　Not Battery Warning

In either case of Battery Warning whose status is not changed or I/O Card.

D0:		Battery Dead

1:  Battery Dead

Set up in case of Memory Card, and when Battery Good status or from Battery Warning to Battery Dead status (an external input signal BVD1A/BVD1B is changed to ”L”)

Set up once, or never be 0 even if returned to Battery Good (“HH”) or Battery Warning.

0:  Not Battery Dead

In either case of Battery Dead whose status is not changed or I/O Card.

.

Bit is reset by following way in this Register (=0).

This register reads when D2 (Global Control Register (Offset: 1Eh)) is set to 0.

1 will be written to each bit when D2 (Global Control Register) is set to 1.

After Card-Put in, D2-D0 will not be sensed between 1ms.

## Card Status Change Interrupt Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 05 | 805 | PPCSI | RW | CIR3 | CIR2 | CIR1 | CIR0 | ECDD | ERCH | EBAW | EBAD | 00h |

D7-D4:	 Card Status Change Interrupt Level

Assigns the interrupt by Card Status Change to PIC interrupt input.

0 0 0 0:  Disable

0 0 0 1:  Disable

0 0 1 0:  Disable

0 0 1 1:  IRQ3

0 1 0 0:  IRQ4

0 1 0 1:  IRQ5

0 1 1 0:  Disable

0 1 1 1:  IRQ7

1 0 0 0:  Disable

1 0 0 1:  IRQ9

1 0 1 0:  IRQ10

1 0 1 1:  IRQ11

1 1 0 0:  IRQ12

1 1 0 1:  Disable

1 1 1 0:  IRQ14

1 1 1 1:  IRQ15

D3:		 Card Detect Sense Enable

1:  Enable

Card Detect enabled.

0:　Disable

D2:		Ready Change Sense Enable

1:  Enable

Ready Change Detect enabled.

0:　Disable

D1:		Battery Warning Sense Enable

1:  Enable

Battery Warning Detect enabled.

0:　Disable

D0:		Battery Dead Sense Enable

1:  Enable

Battery Dead Detect enabled.

0:　Disable

## Window Enable Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 06 | 806 | PPWEN | RW | IOW1 | IOW0 | - | MEW4 | MEW3 | MEW2 | MEW1 | MEW0 | 00h |

D7:		I/O Window 1Enable

D6:		I/O Window 0Enable

1:  Enable

I/O access of address specified by I/O Access I/O Window 1/0 Start Low/High Address Register and I/O Window 1/0 End Low/High Address Register (Offset: 08h-0Fh) will be I/O Access to the Card. In this case, address 25-16 is turned to 0000000000.

0:　Disable

D4:		System Memory Window 4Enable

D3:		System Memory Window 3Enable

D2:		System Memory Window 2Enable

D1:		System Memory Window 1Enable

D0:		System Memory Window 0Enable

1:  Enable

Memory Access of address specified by System Memory Window 4-0 Start Low/High Address  Register and System Memory Window 4-0 End Low/High Address  Register(Offset :30h-33H,28h-2Bh,20h-23h,18h-1Bh,10h-13h) will be Memory Access to the Card. In this case, address 25-12 will be added the value of System Memory Window 4-0 Offset Low/High Address Register (Offset :34h/35h, 2Ch/2Dh, 24h/25h, 1Ch/1Dh, 14h/15h).

0:　Disable

When read in Disable, or read beyond the limits, FFh can be readる(Not answered as a port)

## I/O Window Control Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 07 | 807 | PPWCN | RW | I116 | I1ZW | I1SR | I1DS | IO16 | I0ZW | IOSR | IODS | 00h |

D7:		I/O Window 1 16bit Access Length

Sets up the Number of 16bit I/O Access cycle in I/O Window 1.

1:  4ISA Bus Clock

0:  3ISA Bus Clock

D6:		I/O Window 1 ZeroWait State

Sets up the Number of 8bitsI/O Access in I/O Window 1.

1:   ZeroWait cycle (4ISA Bus Clock)

0:  Standard cycle (6ISA Bus Clock)

D5:		I/O Window 1 16bit Source

1:  External Input Signal

WPA/WPB when I/O Card Mode used.

0:   Register Sets up

Internal Output SignalPCM_I16Z (16bitI/O) enabled (=0) when I1DS is 
set to 1.

D4:		I/O Window 1 Data Size

1:  16bit Access (Enabled when I1SR is set to 0)

0:  8bits Access

D3:		I/O Window 0 16bit Access Length

Sets up 16bitI/O Access Cycle Number in I/O Window 0.

1:  4ISA Bus Clock

0:  3ISA Bus Clock

D2:		I/O Window 0 ZeroWait State

Sets up 8bitI/O Access Cycle Number in I/O Window 0.

1:  Zero Wait cycle (4ISA Bus Clock)

0:  Standard cycle (6ISA Bus Clock)

D1:		I/O Window 0 16bit Source

1:   An external input signal

WPA/WPB when I/O Card Mode used.

0:   Register Sets up

Internal Output SignalPCM_I16Z(16bitI/O) enabled (=0)

when I0DS is set to 1

D0:		I/O Window 0 Data Size

1:  16bit Access (But enabled when I0SR is set to 0)

0:  8bits Access

## I/O Window 0 Start Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 08 | 808 | PPI0S | RW | I0S7 | I0S6 | I0S5 | I0S4 | I0S3 | I0S2 | I0S1 | I0S0 | 00h |

D7-D0:	I/O Window 0 Start Address 7-0

Sets up the start lower 8bits, which have I/O access to the card in I/O Window 0.

## I/O Window 0 Start High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 09 | 809 | PPI0S | RW | I0SF | I0SE | I0SD | I0SC | I0SB | I0SA | I0S9 | I0S8 | 00h |

D7-D0:	I/O Window 0 Start Address 15-8

Sets up the start upper 8bits, which have I/O access to the card in I/O Window 0.

## I/O Window 0 End Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0A | 80A | PPI0E | RW | I0E7 | I0E6 | I0E5 | I0E4 | I0E3 | I0E2 | I0E1 | I0E0 | 00h |

D7-D0:	I/O Window 0 End Address 7-0

Sets up the end lower 8bits, which have I/O access to the card in I/O Window 0.

## I/O Window 0 End High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0B | 80B | PPI0E | RW | I0EF | I0EE | I0ED | I0EC | I0EB | I0EA | I0E9 | I0E8 | 00h |

D7-D0:	I/O Window 0 End Address 15-8

Sets up the end upper 8bits, which have I/O access to the card in I/O Window 0.

## I/O Window 1 Start Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0C | 80C | PPI1S | RW | I1S7 | I1S6 | I1S5 | I1S4 | I1S3 | I1S2 | I1S1 | I1S0 | 00h |

D7-D0:	I/O Window 1 Start Address 7-0

Sets up the start lower 8bits, which have I/O access to the card in I/O Window 1.

## I/O Window 1 Start High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0D | 80D | PPI1S | RW | I1SF | I1SE | I1SD | I1SC | I1SB | I1SA | I1S9 | I1S8 | 00h |

D7-D0:	I/O Window 1 Start Address 15-8

Sets up the start upper 8bits, which have I/O access to the card in I/O Window 1.

## I/O Window 1 End Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0E | 80E | PPI1E | RW | I1E7 | I1E6 | I1E5 | I1E4 | I1E3 | I1E2 | I1E1 | I1E0 | 00h |

D7-D0:	I/O Window 1 End Address 7-0

Sets up the end lower 8bits, which have I/O access to the card in I/O Window 1.

## I/O Window 1 End High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0F | 80F | PPI1E | RW | I1EF | I1EE | I1ED | I1EC | I1EB | I1EA | I1E9 | I1E8 | 00h |

D7-D0:	I/O Window 1 End Address 15-8

Sets up the end upper 8bits, which have I/O access to the card in I/O Window 1.

## System Memory Window 0 Start Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 810 | PPM0S | RW | MS19 | MS18 | MS17 | MS16 | MS15 | MS14 | MS13 | MS12 | 00h |

D7-D0:	System Memory Window 0 Start Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 0.

## System Memory Window 0 Start High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | 811 | PPM0S | RW | M0DS | M0ZW | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7:		System Memory Window 0 Data Size

1:  16bit access

0:  8bit access

D6:		System Memory Window 0 Zero Wait

Sets up the Number of Memory Access cycle.

1:  Zero Wait Cycle

8bit access: 4ISA Clock

16bit access: 2ISA Clock

0:  Standard Cycle (6ISA Clock)

Only 8bit access: Standard cycle

16bit access: Sets up by D7-6 of System Memory Window 0 End
Register (Offset: 13h).

D5-D4:	Unused (READ /WRITE possible)

D3-D0:	System Memory Window 0 Start Address 23-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 0.

## System Memory Window 0 End Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 812 | PPM0E | RW | ME19 | ME18 | ME17 | ME16 | ME15 | ME14 | ME13 | ME12 | 00h |

D7-D0:	System Memory Window 0 End Address 19-12

Sets up the low end order address 8bits, which have memory access to the card in System Memory Window 0.

## System Memory Window 0 End High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 813 | PPM0E | RW | W0S1 | W0S0 | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7-D6:	System Memory Window 0 16bit Memory Access Length

Sets up the cycle number of 16bit Memory Access.

0 0:  3ISA Clock

0 1:  4ISA Clock(1 Wait)

1 0:  5ISA Clock(2 Wait)

1 1:  6ISA Clock(3 Wait)

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 0 End Address 23-20

Sets up the end upper 4bits, which have memory access to the card in System Memory Window 0.

## Memory Window 0 Offset Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 814 | PPM0O | RW | MO19 | MO18 | MO17 | MO16 | MO15 | MO14 | MO13 | MO12 | 00h |

D7-D0:	System Memory Window 0 Offset Address 19-12

Sets up the start lower 8bits added to the address, which have memory access to the card in System Memory Window 0.

## Memory Window 0 Offset High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 815 | PPM0O | RW | E0WP | E0RG | MO25 | MO24 | MO23 | MO22 | MO21 | MO20 | 00h |

D7:		Write Protect Enable

1:  Enable

0:  Disable

Memory write enabled. This bit isn’t affected by memory read.

D6:		Attribute Memory Enable

1:  Enable

Access via System Memory Window will result in attribute memory.
  			An external output signal REGAZ/REGBZ enabled (=“L”)

0:  Disable

Access via System Memory Window will result in common memory.
 			An external output signal REGAZ/REGBZ canceled (=“H”).

D5-D0:	System Memory Window 0 Offset Address 25-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 0.

## Additional General Control Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 816 | PPACN | RW | VS2 | VS1 | SWD | - | - | - | CRR | - | 00h |

D7:		Voltage Sense 2

Indicates the status of an external input signal VS2A/VS2B when D1 (PC-Card IF Function Register (Offset: 3Eh)) is set to 1.

D6:		Voltage Sense 1

Indicates the status of an external input signal VS1A/VS2B when D1 (PC-Card IF Function Register (Offset: 3Eh)) is set to 1.

D5:		Software Card Sense Enable

1:　Enable

The Card is detected by Software. Sets up 1 in D3 (Card Status Change

Register (Offset: 04h)).

0:　Disable

D4-D2:	Unused (READ / WRITE POSSIBLE)

D1:		 Card Remove Reset

1:　 Register Reset

The Card is removed and then, reset the Offset: 00h-3Fh register without

following Register.

Register not reset

ID& Revision (Offset: 00h) all bits

Interface Status (Offset: 01h) all bits

Power Control (Offset: 02h) all bits

Interrupt and General Control (Offset: 03h) 4D

Card Status Change (Offset: 04h) all bits

Card Status Change interrupt (Offset: 05h) all bits

Window Enable (Offset: 06h) 5D

General Control (Offset: 16h) all bits

Global Control (Offset: 1Eh) all bits

PC-Card IF Card Timing (Offset: 3Bh) all bits

PC-Card IF Function Control (Offset: 3Eh) all bits

Toshiba Hidden (Offset: 3Fh)all bits

0:　No Register Reset

D0:		Unused (READ / WRITE POSSIBLE)

## System Memory Window 1 Start Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | 818 | PPM1S | RW | MS19 | MS18 | MS17 | MS16 | MS15 | MS14 | MS13 | MS12 | 00h |

D7-D0:	System Memory Window 1 Start Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 1.

## System Memory Window 1 Start High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | 819 | PPM1S | RW | M1DS | M1ZW | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7:		System Memory Window 1 Data Size

1:  16bit Access

0:  8bits Access

D6:		System Memory Window 1 Zero Wait

Sets up the cycle number of Memory Access.

1:  Zero Wait Cycle

8bit access: 4ISA Clock

16bit access: 2ISA Clock

0:  Standard Cycle (6ISA Clock)

Only 8bit access: Standard cycle

16bit access: Sets up by D7-6 of System Memory Window 1 End
Register (Offset: 1Bh).

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 1 Start Address 23-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 1.

## System Memory Window 1 End Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1A | 81A | PPM1E | RW | ME19 | ME18 | ME17 | ME16 | ME15 | ME14 | ME13 | ME12 | 00h |

D7-D0:	System Memory Window 1 End Address 19-12

Sets up the end lower 8bits, which have memory access to the card in System Memory Window 1.

## System Memory Window 1 End High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1B | 81B | PPM1E | RW | W1S1 | W1S0 | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7-D6:	System Memory Window 1 16bitMemory Access Length

16bitMemory Access の cycle NumberをSets upする.

0 0:  3ISA Clock

0 1:  4ISA Clock(1 Wait)

1 0:  5ISA Clock(2 Wait)

1 1:  6ISA Clock(3 Wait)

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 1 End Address 23-20

Sets up the end upper 4bits, which have memory access to the card in System Memory Window 1.

## Memory Window 1 Offset Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1C | 81C | PPM1O | RW | MO19 | MO18 | MO17 | MO16 | MO15 | MO14 | MO13 | MO12 | 00h |

D7-D0:	System Memory Window 1 Offset Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 0.

## Memory Window 1 Offset High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1D | 81D | PPM1O | RW | E1WP | E1RG | MO25 | MO24 | MO23 | MO22 | MO21 | MO20 | 00h |

D7:		Write Protect Enable

1:  Enable

0:  Disable

Memory write enabled. This bit isn’t affected by memory read.

D6:		Attribute Memory Enable

1:  Enable

Access via System Memory Window will result in attribute memory.
  			An external output signal REGAZ/REGBZ enabled (=“L”)

0:  Disable

Access via System Memory Window will result in common memory.
 			An external output signal REGAZ/REGBZ canceled (=“H”).

D5-D0:	System Memory Window 1 Offset Address 25-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 0.

## Global Control Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1E | 81E | PPGCN | RW | - | - | - | - | - | RCSC | - | - | 00h |

D7-D3:	Unused (READ / WRITE POSSIBLE)

D2:		 Card Status Change Register Initialized

1:　Initialized by Write Back.

The bit is reset by Write 1 to D1 (Card Status Change

Register (Offset: 04h))(=0).

0:　Initialized by Read

All bits (Card Status Change Register )are reset (=0) when Card Status Change Register will be read, and interrupt by Card Status Change Register is canceled.

This register is a register common to Slot A and Slot B. If accessed from either, accessed to the same register.

## System Memory Window 2 Start Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 820 | PPM2S | RW | MS19 | MS18 | MS17 | MS16 | MS15 | MS14 | MS13 | MS12 | 00h |

D7-D0:	System Memory Window 2 Start Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 2.

## System Memory Window 2 Start High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | 821 | PPM2S | RW | M2DS | M2ZW | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7:		System Memory Window 2 Data Size

1:  16bit Access

0:  8bits Access

D6:		System Memory Window 2 Zero Wait

Sets up the cycle number of Memory Access.

1:  Zero Wait Cycle

8bit access: 4ISA Clock

16bit access: 2ISA Clock

0:  Standard Cycle (6ISA Clock)

Only 8bit access: Standard cycle

16bit access: Sets up by D7-6 of System Memory Window 2 End
Register (Offset: 23h).

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 2 Start Address 23-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 2.

## System Memory Window 2 End Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | 822 | PPM2E | RW | ME19 | ME18 | ME17 | ME16 | ME15 | ME14 | ME13 | ME12 | 00h |

D7-D0:	System Memory Window 2 End Address 19-12

Sets up the end lower 8bits, which have memory access to the card in System Memory Window 2.

## System Memory Window 2 End High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 823 | PPM2E | RW | W2S1 | W2S0 | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7-D6:	System Memory Window 2 16bitMemory Access Length

Sets up the cycle number of 16bitMemory Access.

0 0:  3ISA Clock

0 1:  4ISA Clock(1 Wait)

1 0:  5ISA Clock(2 Wait)

1 1:  6ISA Clock(3 Wait)

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 2 End Address 23-20

Sets up the end upper 4bits, which have memory access to the card in System Memory Window 2.

## Memory Window 2 Offset Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 824 | PPM2O | RW | MO19 | MO18 | MO17 | MO16 | MO15 | MO14 | MO13 | MO12 | 00h |

D7-D0:	System Memory Window 2 Offset Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 0.

## Memory Window 2 Offset High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 825 | PPM2O | RW | E2WP | E2RG | MO25 | MO24 | MO23 | MO22 | MO21 | MO20 | 00h |

D7:		Write Protect Enable

1:  Enable

0:  Disable

Memory write enabled. This bit isn’t affected by memory read.

D6:		Attribute Memory Enable

1:  Enable

Access via System Memory Window will result in attribute memory.
  			An external output signal REGAZ/REGBZ enabled (=“L”)

0:  Disable

Access via System Memory Window will result in common memory.
 			An external output signal REGAZ/REGBZ canceled (=“H”).

D5-D0:	System Memory Window 2 Offset Address 25-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 0.

## System Memory Window 3 Start Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 828 | PPM3S | RW | MS19 | MS18 | MS17 | MS16 | MS15 | MS14 | MS13 | MS12 | 00h |

D7-D0:	Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 3.

## System Memory Window 3 Start High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29 | 829 | PPM3S | RW | M3DS | M3ZW | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7:		Data Size

1:  16bit access

0:  8bits access

D6:		Zero Wait State

Sets up the Number of Memory Access.

1:  Zero Wait Cycle

8bit access: 4ISA Clock

16bit access: 2ISA Clock

0:  Standard Cycle (6ISA Clock)

Only 8bit access: Standard cycle

16bit access: Sets up by D7-6 of System Memory Window 3 End
Register (Offset: 2Bh).

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	Address 23-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 3.

## System Memory Window 3 End Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2A | 82A | PPM3E | RW | ME19 | ME18 | ME17 | ME16 | ME15 | ME14 | ME13 | ME12 | 00h |

D7-D0:	System Memory Window 3 End Address 19-12

Sets up the end lower 8bits, which have memory access to the card in System Memory Window 3.

## System Memory Window 3 End High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2B | 82B | PPM3E | RW | W3S1 | W3S0 | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7-D6:	System Memory Window 3 16bitMemory Access Length

16bitMemory Access の cycle NumberをSets upする.

0 0:  3ISA Clock

0 1:  4ISA Clock(1 Wait)

1 0:  5ISA Clock(2 Wait)

1 1:  6ISA Clock(3 Wait)

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 3 End Address 23-20

Sets up the end upper 4bits, which have memory access to the card in System Memory Window 3.

## Memory Window 3 Offset Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2C | 82C | PPM3O | RW | MO19 | MO18 | MO17 | MO16 | MO15 | MO14 | MO13 | MO12 | 00h |

D7-D0:	System Memory Window 3 Offset Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 0.

## Memory Window 3 Offset High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2D | 82D | PPM3O | RW | E3WP | E3RG | MO25 | MO24 | MO23 | MO22 | MO21 | MO20 | 00h |

D7:		Write Protect Enable

1:  Enable

0:  Disable

Memory write enabled. This bit isn’t affected by memory read.

D6:		Attribute Memory Enable

1:  Enable

Access via System Memory Window will result in attribute memory.
  			An external output signal REGAZ/REGBZ enabled (=“L”)

0:  Disable

Access via System Memory Window will result in common memory.
 			An external output signal REGAZ/REGBZ canceled (=“H”).

D5-D0:	System Memory Window 3 Offset Address 25-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 0.

## System Memory Window 4 Start Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 830 | PPM4S | RW | MS19 | MS18 | MS17 | MS16 | MS15 | MS14 | MS13 | MS12 | 00h |

D7-D0:	System Memory Window 4 Start Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 4.

## System Memory Window 4 Start High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 831 | PPM4S | RW | M4DS | M4ZW | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7:		System Memory Window 4 Data Size

1:  16bit Access

0:  8bits Access

D6:		System Memory Window 4 Zero Wait

Sets up the cycle number of Memory Access.

1:  Zero Wait Cycle

8bit access: 4ISA Clock

16bit access: 2ISA Clock

0:  Standard Cycle (6ISA Clock)

Only 8bit access: Standard cycle

16bit access: Sets up by D7-6 of System Memory Window 4 End
Register (Offset: 33h).

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 4 Start Address 23-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 4.

## System Memory Window 4 End Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 832 | PPM4E | RW | ME19 | ME18 | ME17 | ME16 | ME15 | ME14 | ME13 | ME12 | 00h |

D7-D0:	System Memory Window 4 End Address 19-12

Sets up the end lower 8bits, which have memory access to the card in System Memory Window 4.

## System Memory Window 4 End High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | 833 | PPM4E | RW | W4S1 | W4S0 | - | - | MS23 | MS22 | MS21 | MS20 | 00h |

D7-D6:	System Memory Window 4 16bitMemory Access Length

Sets up the cycle number of 16bit Memory Access.

0 0:  3ISA Clock

0 1:  4ISA Clock(1 Wait)

1 0:  5ISA Clock(2 Wait)

1 1:  6ISA Clock(3 Wait)

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3-D0:	System Memory Window 4 End Address 23-20

Sets up the end upper 4bits, which have memory access to the card in System Memory Window 4.

## Memory Window 4 Offset Low Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 | 834 | PPM4O | RW | MO19 | MO18 | MO17 | MO16 | MO15 | MO14 | MO13 | MO12 | 00h |

D7-D0:	System Memory Window 4 Offset Address 19-12

Sets up the start lower 8bits, which have memory access to the card in System Memory Window 0.

## Memory Window 4 Offset High Address Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | 835 | PPM4O | RW | E4WP | E4RG | MO25 | MO24 | MO23 | MO22 | MO21 | MO20 | 00h |

D7:		Write Protect Enable

1:  Enable

0:  Disable

Memory write enabled. This bit isn’t affected by memory read.

D6:		Attribute Memory Enable

1:  Enable

Access via System Memory Window will result in attribute memory.
  			An external output signal REGAZ/REGBZ enabled (=“L”)

0:  Disable

Access via System Memory Window will result in common memory.
 	An external output signal REGAZ/REGBZ canceled (=“H”).

D5-D0:	System Memory Window 4 Offset Address 25-20

Sets up the start upper 4bits, which have memory access to the card in System Memory Window 0.

## PC-Card IF Card Timing Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3B | 83B | PPCTM | RW | INPZ | - | SCY1 | SCY0 | HCY3 | HCY2 | HCY1 | HCY0 | 00h |

D7:		 Input Answer

Indicates INP an external input signal CKAZ/INPCKBZ.

D6:		Unused (READ / WRITE POSSIBLE)

D5-D4:	Set Up Cycle Number

Sets up Wait Number inserted to Set Up Cycle of Card Access.

0 0:　0PCI Clock

0 1:　1PCI Clock

1 0:　2PCI Clock

1 1:　3PCI Clock

D3-D0:	Hold Cycle Number

Sets up Wait Number inserted to Hold Cycle of Card Access.

0 0 0 0:　 0PCI Clock

0 0 0 1:　 1PCI Clock

0 0 1 0:　 2PCI Clock

・

・

1 1 1 1:　15PCI Clock

This Register enabled when D3 (PC-Card IF Function Control

Register (Offset: 3Eh)) is set to 1,.

## Audio/Video Switching Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3C | 83C | PPAVS | RW | - | - | - | - | - | - | ACNT | VCNT | 00h |

D7-D2:	Unused (READ / WRITE POSSIBLE)

D1:		AUDIO Control

1:  ZV port Mode

Fixed that an external input signal BVD2A/BVD2B(SPKRZ) and

INPCKAZ/INPCKBZ are 1.

0:  PC-Card16 Mode

D0:		VIDEO Control

1:  ZV port Mode

PC-Card IF doesn’t drive because PC Card Address (external I/O

SignalCADA25-7/CADB25-7) is used as a signal for ZV port

0:  PC-Card 16 Mode

D1-D0 can read the status of ZV port (1:Zvport is running) when D1 (MISC2 Register (Config: AEh)) is set to 1 and will be changed because of being connected with D0 (ZV Control Register (Config: 9Ch)) when D0 is changed .

## PC-Card IF Function Control Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3E | 83E | PPTFN | RW | - | PCBE | - | - | CTE | - | VSSE | CPE | 00h |

D7:		Unused (READ / WRITE POSSIBLE)

D6:		 Slot Power Supply  Control Buffer Enable

1:  Enable

Card VPP Control Signal turned to ”Z” when Buffer Off (an external input signal BUFOFFZ=“L”)

0:  Disable

D5-D4:	Unused (READ / WRITE POSSIBLE)

D3:		 Card Timing Enable

1:  Enable

Sets up Card Access Timing will be set the value determined in PC-Card IF Card Timing Register (Offset: 3Bh).

0:  Disable

D2:		Unused (READ / WRITE POSSIBLE)

D1:		Voltage Sense Enable

1:  Enable

Voltage Sense enabled in case of D7-6 (General Control Register (Offset:

16h)).

0:  Disable

D0:		 Card3V Power Supply  Enable

1:  Enable

3.3V in Card Power Supply  enabled in case of D3 (Power Control Register (Offset: 02h)) .

0:  Disable

## Toshiba Hidden Register

| Offset | Offset | NAME | RW | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3F | 83F | PPHDN | RW | - | - | - | - | - | PIRQ | CRMD | BUFO | 00h |

D2:		 Interrupt Request Signal

1:  Not inverted

Interrupt Signal (BSYAZ/BSYBZ) from the Card input to PCI.

Pulse Mode Interrupt.

0:  Inverted

Interrupt Signal from the Card inverted and input to PIC.

Level Mode Interrupt.

D1:		 Card Remove Sense

1:   Card Remove

Remove detect (put in Sense disabled) function is same as D3 (Card

Status Change Register (Offset: 04h)).

0:　 No Card Remove

This bit can only read. Bit0 is reset by Read.

D0:		 Slot Buffer Off Control Disable

1:  Disable

0:  Enable

Buffer Off in Slot of PC Card.

This Register can have access when D5 (Slot Control Register (Config: A0h))

is set to.

# Card Bus Register

## Socket Event Register

| Offset |  | NAME | RW | D31 |  |  | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 |  | PPSKE | R | - |  |  |  | PWCY | CCD2 | CCD1 | WKUP | 00h |

D3:		Power Cycle

Indicates D3 (Socket Present State Register (Offset: 008h)) will change or not.

1:　Changed

0:  Not changed

D2:	  	CCD2Z

Indicates D2 (Socket Present State Register (Offset: 008h)) will change or not.

1:　Changed

0:  Not changed

D1:		CCD1Z

Indicates D1 (Socket Present State Register (Offset: 008h)) will change or not.

1:　Changed

0:  Not changed

D0:  		CSTSCHG/WAKEUP

Indicates D0 (Socket Present State Register (Offset: 008h)) will change or not.

1:　Changed

0:  Not changed

D3-D0 will initiated by Write 1

This Register canceled when Card Detect way depends on Software

(D0 (Card Detect Control (Config: A3h) is 1)).

## Socket  Mask  Register

| Offset |  | NAME | RW | D31 |  |  | D4 | D3 | D2 | D1 | D0 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 004 |  | PPSKM | RW | - |  |  |  | MPWC | MCD2 | MCD1 | MWKU | 00h |

D3:		Power Cycle Mask Disable

1:　Disable

0:  Enable

Interrupt by Power Supply as a source of Status Change Interrupt not

masked( Interrupt will occur).

D2-D1:  	 Card Detect Interrupt Mask Disable

0 0:　Change Interrupt by Card Detect masked

0 1:  Canceled ( Change Interrupt not occur)

1 0:　Canceled ( Change Interrupt not occur)

1 1:　Change Interrupt by Card Detect not masked

D0:  		STSCHG/WAKEUP Mask Disable

1:　Disable

0:  Enable

Interrupt from an external input signal BVD1A/BVD1B (CSTSCHG) as a source of Change Interrupt not masked ( Interrupt occurred).

## Socket Present State Register

| Config |  | NAME | RW | D31 | D30 | D29 |  | D28 | D27 |  |  |  | D16 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 008 |  | PPSPS | R | - | - | - |  | - | - |  |  |  |  | 3000h |
|  |  |  |  | D15 | D14 | D13 | D12 |  | D11 | D10 |  | D9 | D8 | PCLR |
|  |  |  |  | - | - | CDYV | CDXY |  | CD3V | CD5V |  | BADV | DLOS | 00h |
|  |  |  |  | D7 | D6 | D5 | D4 |  | D3 |  | D2 | D1 | D0 | PCLR |
|  |  |  |  | NCAD | CINT | CARD | PC16 |  | PWCY |  | CCD2 | CCD1 | STCH | 00h |

D29-D28:	Unused (1 fixed)

D13:		YV Card

1:  YV Card

Indicates the Card detected by VS2AZ/VS2BZ, VS1AZ/VS1BZ,

External output signals CD2AZ/CD2BZ, CD1AZ/CD1BZ has YV function.

0:  Not YV Card

D12:		XV Card

1:  XV Card

Indicates the Card detected by VS2AZ/VS2BZ, VS1AZ/VS1BZ,

External output signals CD2AZ/CD2BZ, CD1AZ/CD1BZ has XV function.

0:  Not XV Card

D11:		3V Card

1:  3V Card

Indicates the Card detected by VS2AZ/VS2BZ, VS1AZ/VS1BZ,

External output signals CD2AZ/CD2BZ, CD1AZ/CD1BZ has 3V function.

0:  Not 3V Card

D10:		5V Card

1:  5V Card

Indicates the Card detected by VS2AZ/VS2BZ, VS1AZ/VS1BZ,

External output signals CD2AZ/CD2BZ, CD1AZ/CD1BZ has 5V function.

0:  Not 5V Card

D9:		Socket Power Supply Unusual

1:   Power supply unusual

Indicates the Card is beyond the limits detected by VS2AZ/VS2BZ, External output signals VS1AZ/VS1BZ, CD2AZ/CD2BZ, CD1AZ/CD1BZ.

0:   Power supply normal

D8:		 Data Lost

1:   Data disappeared

Indicates the data disappearance when the Card removed.

0:　 Data normal

D7:		Unclear Card

1:  Support Card put in.

0:　Non-Support Card put in.

D6:		Card Bus Interrupt

1:　 Interrupt occurred

Interrupt from the Card asserted.

0:　 Interrupt not occur

D5:		Card Bus Card

1:  Put in

Sets up when CardBus Card put in.

0:  Not put in

D4:		R2 Card

1:  Put in

Sets up when PC-Card16 put in.

0:  Not put in

D3:		Power Cycle

1:　 Power Supply  balanced

0:　 Power Supply  unbalanced

D2:		 Card Detect 2

Indicates CD2AZ/CD2BZ status.

D1:		 Card Detect 1

Indicates CD1AZ/CD1BZ status.

D0:		 Change

Indicates BVD1A/BVD1B(CSTSCHG) status.

## Socket Force Register

| Config |  | NAME | RW | D31 |  |  |  |  |  |  |  |  | D16 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00C |  | PPSKF | RW | - |  |  |  |  |  |  |  |  |  | 0000h |
|  |  |  |  | D15 | D14 | D13 | D12 |  | D11 | D10 |  | D9 | D8 | PCLR |
|  |  |  |  | - | CVST | CDYV | CDXY |  | CD3V | CD5V |  | BADV | DLOS | 00h |
|  |  |  |  | D7 | D6 | D5 | D4 |  | D3 |  | D2 | D1 | D0 | PCLR |
|  |  |  |  | NCAD | CINT | CARD | PC16 |  | PWCY |  | CCD2 | CCD1 | STCH | 00h |

This Register is for Debug.

D14:		CVS Test Enable

1:　Enable

Detects again by VS2AZ/VS2BZ, VS1AZ/VS1BZ, CD2AZ/CD2BZ, CD1AZ/ CD1BZ, and sets up the Card function in D11-10 (Socket Present State Register(Offset: 08h)).

0:　Disable

D13:		YV Card Enable

Sets up to D12 (Socket Present State Register).

D12:		XV Card Enable

Sets up to D12 (Socket Present State Register).

D11:		3V Card Enable

Sets up to D11 (Socket Present State Register).

D10:		5V Card Enable

Sets up to D10 (Socket Present State Register).

D9:		Socket Power Supply  Unusual Enable

Sets up to D9 (Socket Present State Register).

D8:		Data Lost Enable

Sets up to D8 (Socket Present State Register).

D7:		Unclear Card Enable

Sets up to D7 (Socket Present State Register).

D6:		Card Bus Interrupt Enable

Sets up to D6 (Socket Present State Register).

D5:		Card Bus Card Enable

Sets up to D5 (Socket Present State Register).

D4:		R2 Card Enable

Sets up to D4 (Socket Present State Register).

D3:		Power Cycle Enable

Sets up to D3 (Socket Present State Register).

D2:		Card Detect 2Enable

Sets up to D2 (Socket Present State Register).

D1:		Card Detect 1Enable

Sets up to D1 (Socket Present State Register).

D0:		Status Change Enable

Sets up to D0 (Socket Present State Register).

## Power Control Register

| Offset |  | NAME | RW | D31 |  |  |  |  |  |  |  | D8 | PCLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 010 |  | PPPWC | RW | - |  |  |  |  |  |  |  |  | 00h |
|  |  |  |  | D7 | D6 | D5 | D4 |  | D3 | D2 | D1 | D0 | PCLR |
|  |  |  |  | CKRN | VCC2 | VCC1 | VCC0 |  | - | VPP2 | VPP1 | VPP0 | 00h |

This Register controls Vpp supplied to the socket and Vcc Power Supply .

D7:		Card Bus CLOCKRUN Enable

1:  Enable

CLOCKRUN function in Card Bus function enabled, but D29 (Card Bus Socket Register Control Register (Config:A4h)) has priority.

0:  Disable

D6-D4:	 Card VCC Control

0 0 0:   Power Supply  Off

0 0 1:  -

0 1 0:  5V

0 1 1:  3V

1 X X:  -

D2-D0:	 Card VPP Control

0 0 0:   Power Supply  Off

0 0 1:  12V

0 1 0:   5V

0 1 1:   3V

1 X X:  -